ניתוח אמינות השוואתי

MIL-STD-217F Notice 2  •  נתוני יצרנים (FIT Database)

פרויקט: SPWR Board Rev3

מספר מסמך: REL-2026-001

הוכן על ידי: מהנדס אמינות

גרסה: A

תאריך: 20/06/2026

# 0  תוכן עניינים

סעיף 1  –  מבוא

סעיף 2  –  שיטות החישוב

סעיף 3  –  הנחות וגבולות

סעיף 4  –  סימוכין

סעיף 5  –  תוצאות – ניתוח MIL-STD-217F

סעיף 6  –  תוצאות – ניתוח נתוני יצרנים

סעיף 7  –  טבלת השוואה

סעיף 8  –  MTBF מסכם ומסקנות

# 1  מבוא

מסמך זה מציג ניתוח אמינות כפול של המעגל האלקטרוני בהתאם לשתי מתודולוגיות משלימות:

• MIL-STD-217F Notice 2 – תקן הצבאי האמריקאי לחיזוי אמינות של ציוד אלקטרוני, המשתמש במודלים פיזיקליים מפורטים עם גורמי סביבה, איכות וטמפרטורה.

• נתוני יצרנים (Manufacturer FIT Database) – קצבי כשל שפורסמו על ידי יצרני הרכיבים (TI, Analog Devices, Murata, Vishay, Micron ועוד) בדוחות אמינות ו-Qualification Reports, עם תיקון טמפרטורה לפי מודל Arrhenius.

השוואת שתי המתודולוגיות מאפשרת הערכה מאוזנת יותר של אמינות המערכת ומדגישה רכיבים שבהם קיים פער משמעותי בין הגישות.

# 2  שיטות החישוב

## 2.1  MIL-STD-217F Notice 2

הנוסחה הכללית לחישוב קצב הכשל הצפוי (λ_p) ביחידות כשלים / 10⁶ שעות:

λ_p = λ_b × π_T × π_Q × π_E × [גורמים נוספים לפי סוג הרכיב]

• λ_b – קצב כשל בסיסי – ספציפי לסוג ולמבנה הרכיב

• π_T – גורם טמפרטורה – מבוסס על Arrhenius עם אנרגיית הפעלה Ea

• π_Q – גורם איכות – נגזר מרמת ההסמכה של הרכיב

• π_E – גורם סביבה – עומסים מכניים, רטט, לחות, אקלים

• π_R/π_CV – גורמים נוספים – עצמה/קיבול/עקה בהתאם לסוג הרכיב

• מעגלים משולבים: סעיף 5.1 –  (C₁×π_T + C₂×π_E) × π_Q × π_L

• טרנזיסטורים: סעיף 6   –  λ_b × π_T × π_A × π_S × π_Q × π_E

• דיודות: סעיף 7   –  λ_b × π_T × π_S × π_C × π_Q × π_E

• נגדים (Film): סעיף 9   –  λ_b × π_T × π_R × π_Q × π_E

• קבלים (Ceramic): סעיף 10  –  λ_b × π_CV × π_T × π_Q × π_E

• סלילים / שנאים: סעיף 11  –  λ_b × π_T × π_Q × π_E

• מחברים: סעיף 15  –  λ_b × π_E × π_Q × π_P

• קריסטלים / אוסק': סעיף 18  –  λ_b × π_T × π_Q × π_E

## 2.2  נתוני יצרנים – FIT Database

נתוני ה-FIT (Failures in Time = כשלים / 10⁹ שעות) נלקחו מדוחות Qualification ומ-Reliability Handbooks שפרסמו יצרני הרכיבים. תיקון הטמפרטורה מבוסס על נוסחת Arrhenius:

FIT(T) = FIT₂₅ × exp[ Ea/kB × (1/T_ref − 1/T_use) ]

• FIT₂₅ – קצב כשל מפורסם על ידי היצרן ב-25°C

• Ea – אנרגיית הפעלה [eV] – אופיינית לסוג הרכיב

• kB – קבוע בולצמן: 8.617×10⁻⁵ eV/K

• T_ref – 298.15 K (25°C)

• T_use – טמפרטורת הפעלה [K] = T_ambient + 273.15

# 3  הנחות וגבולות

• סביבת הפעלה: GF – קרקעי, קבוע

• טמפרטורת סביבה מרבית: 40.0 °C

• יחס עקה (Stress Ratio V/Vrated): 50%

• רמת איכות רכיבים (MIL-217F): C – Commercial / Plastic (C)

• π_L (Learning Factor) – הונחה תקופת ייצור > 2 שנים (π_L = 0.5)

• חישוב MTBF מניח קצב כשל קבוע (Constant Failure Rate – CFR, אזור Flat Bathtub)

• לא נכללות בחישוב: תקופת Burn-In (ינקות) וסוף חיים (Wear-Out)

• רכיבים ללא מידע זוהו ע"פ ה-Description וסווגו לקטגוריה המתאימה

• נתוני יצרנים מבוססים על תנאי בדיקה של 25°C, עם תיקון Arrhenius לטמפ' הפעלה

# 4  סימוכין

[1]  MIL-STD-217F Notice 2, "Reliability Prediction of Electronic Equipment," DoD, 1995.

[2]  MIL-HDBK-338B, "Electronic Reliability Design Handbook," 1998.

[3]  JEDEC JEP122H, "Failure Mechanisms and Models for Semiconductor Devices," 2016.

[4]  Vishay Resistive Systems – "CRCW Series Reliability Data," 2023.

[5]  KEMET – "MLCC Reliability Report," 2023.

[6]  Murata Manufacturing – "BLM/DLW/GRM Series Reliability Data," 2023.

[7]  Analog Devices – "LTM/LT Series Reliability Qualification Report," 2023.

[8]  Texas Instruments – "Quality and Reliability Handbook," 2023.

[9]  Micron Technology – "Memory Product Reliability Report," 2023.

[10] Infineon Technologies – "FRAM CY15 Series Reliability Data," 2023.

[11] CTS Corporation – "Oscillator/TCXO Qualification Report," 2023.

[12] Intel/Altera – "Agilex Series Reliability Qualification," 2023.

# 5  תוצאות – ניתוח MIL-STD-217F

סביבה: GF | טמפרטורה: 40.0°C | איכות: C | עקה: 50%

| תיאור / Part# | יצרן | סוג | כמ' | π_T | π_Q | π_E | מודל | λ_p יחידה (כשלים/10⁶שע) | λ_p סה"כ (כשלים/10⁶שע) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SMT CHIP JUMPER RESISTOR 0 Ohm,0.06 CRCW04020000ZSTD | VISHAY RESISTIVE SYSTEMS | RESISTOR | 41 | 1.327 | 10 | 1.00 | §9 Resistor | 0.00080 | 0.03264 |
| SMT CHIP RESISTOR 1 KOhm,1%,0.063W, CRCW04021K00FKTD | VISHAY RESISTIVE SYSTEMS | RESISTOR | 26 | 1.327 | 10 | 1.00 | §9 Resistor | 0.00080 | 0.02070 |
| SMT CHIP RESISTOR 75 Ohm,1%,0.25W,T CRCW120675R0FKTA | VISHAY SEMICONDUCTOR | RESISTOR | 2 | 1.327 | 10 | 1.00 | §9 Resistor | 0.00080 | 0.00159 |
| SMT CHIP RESISTOR 100 KOhm,1%,0.063 CRCW0402100KFKTD | VISHAY RESISTIVE SYSTEMS | RESISTOR | 20 | 1.327 | 10 | 1.00 | §9 Resistor | 0.00080 | 0.01592 |
| SMT CHIP RESISTOR 49.9 Ohm,1%,0.063 CRCW040249R9FKTD | VISHAY RESISTIVE SYSTEMS | RESISTOR | 4 | 1.327 | 10 | 1.00 | §9 Resistor | 0.00080 | 0.00318 |
| SMT CHIP RESISTOR 200 KOhm,1%,0.063 CRCW0402200KFKTD | VISHAY RESISTIVE SYSTEMS | RESISTOR | 1 | 1.327 | 10 | 1.00 | §9 Resistor | 0.00088 | 0.00088 |
| SMT CHIP RESISTOR 24.9 KOhm,1%,0.06 CRCW040224K9FKTD | VISHAY RESISTIVE SYSTEMS | RESISTOR | 1 | 1.327 | 10 | 1.00 | §9 Resistor | 0.00080 | 0.00080 |
| SMT CHIP RESISTOR 48.7 KOhm,1%,0.06 CRCW040248K7FKTD | VISHAY INTERTECHNOLOGY IN | RESISTOR | 2 | 1.327 | 10 | 1.00 | §9 Resistor | 0.00080 | 0.00159 |
| CHIP RESIS 56.2KOHM 1% .063W T&R CRCW040256K2FKTD | VISHAY RESISTIVE SYSTEMS | IC | 2 | 0.374 | 14 | 1.00 | §5.1 Monolithic | 0.25900 | 0.51800 |
| SMT CHIP RESISTOR 402 Ohm,1%,0.063W CRCW0402402RFKTD | VISHAY DALE | RESISTOR | 2 | 1.327 | 10 | 1.00 | §9 Resistor | 0.00080 | 0.00159 |
| SMT CHIP RESISTOR 40.2 KOhm,1%,0.06 CRCW040240K2FKTD | VISHAY DALE | RESISTOR | 2 | 1.327 | 10 | 1.00 | §9 Resistor | 0.00080 | 0.00159 |
| CHIP RESIS 150 OHM 1% .063W T&R CRCW0402150RFKTD | VISHAY RESISTIVE SYSTEMS | IC | 3 | 0.374 | 14 | 1.00 | §5.1 Monolithic | 0.25900 | 0.77700 |
| SMT CHIP RESISTOR 2.21 KOhm,1%,0.06 CRCW04022K21FKTD | VISHAY RESISTIVE SYSTEMS | RESISTOR | 2 | 1.327 | 10 | 1.00 | §9 Resistor | 0.00080 | 0.00159 |
| SMT CHIP RESISTOR 4.7 KOhm,1%,0.063 CRCW04024K70FKTD | VISHAY INTERTECHNOLOGY IN | RESISTOR | 3 | 1.327 | 10 | 1.00 | §9 Resistor | 0.00080 | 0.00239 |
| SMT CHIP RESISTOR 137 KOhm,1%,0.063 CRCW0402137KFKTD | VISHAY INTERTECHNOLOGY IN | RESISTOR | 6 | 1.327 | 10 | 1.00 | §9 Resistor | 0.00088 | 0.00526 |
| SMT CHIP CAPACITOR 10000pF,10%,50V, C0402C103K5RALTU | KEMET ELECTRONICS SA | CAPACITOR | 6 | 1.327 | 10 | 1.00 | §10 Capacitor | 0.00221 | 0.01329 |
| CHIP CAP 0.1UF 10% 16V X7R T&R GRM033Z71C104KE14D | MURATA | CAPACITOR | 199 | 1.327 | 10 | 1.00 | §10 Capacitor | 0.00335 | 0.66705 |
| SMT CHIP RESISTOR 10 KOhm,1%,0.063W CRCW040210K0FKED | VISHAY RESISTIVE SYSTEMS | RESISTOR | 51 | 1.327 | 10 | 1.00 | §9 Resistor | 0.00080 | 0.04060 |
| SMT CHIP RESISTOR 2 KOhm,1%,0.063W, CRCW04022K00FKED | VISHAY RESISTIVE SYSTEMS | RESISTOR | 1 | 1.327 | 10 | 1.00 | §9 Resistor | 0.00080 | 0.00080 |
| SMT CHIP RESISTOR 499 Ohm,1%,0.063W CRCW0402499RFKED | VISHAY RESISTIVE SYSTEMS | RESISTOR | 7 | 1.327 | 10 | 1.00 | §9 Resistor | 0.00080 | 0.00557 |
| SMT CHIP RESISTOR 49.9 KOhm,1%,0.06 CRCW040249K9FKED | VISHAY RESISTIVE SYSTEMS | RESISTOR | 1 | 1.327 | 10 | 1.00 | §9 Resistor | 0.00080 | 0.00080 |
| SMT CHIP RESISTOR 42.2 KOhm,1%,0.06 CRCW040242K2FKED | VISHAY RESISTIVE SYSTEMS | RESISTOR | 2 | 1.327 | 10 | 1.00 | §9 Resistor | 0.00080 | 0.00159 |
| SMT CHIP RESISTOR 6.19 KOhm,1%,0.06 CRCW04026K19FKED | VISHAY INTERTECHNOLOGY IN | RESISTOR | 1 | 1.327 | 10 | 1.00 | §9 Resistor | 0.00080 | 0.00080 |
| Resistor SMT, 3.01Kohm, 1%, 0.063W, CRCW04023K01FKED | VISHAY RESISTIVE SYSTEMS | RESISTOR | 2 | 1.327 | 10 | 1.00 | §9 Resistor | 0.00080 | 0.00159 |
| SMT CHIP RESISTOR 23.2 Kohm,1%,0.06 CRCW040223K2FKTD | VISHAY DALE | RESISTOR | 3 | 1.327 | 10 | 1.00 | §9 Resistor | 0.00080 | 0.00239 |
| SMT CHIP RESISTOR 8.45 KOhm,1%,0.06 CRCW04028K45FKTD | VISHAY DALE | RESISTOR | 1 | 1.327 | 10 | 1.00 | §9 Resistor | 0.00080 | 0.00080 |
| SMT CHIP RESISTOR 10.7 Kohm,1%,0.06 CRCW040210K7FKTD | VISHAY DALE | RESISTOR | 2 | 1.327 | 10 | 1.00 | §9 Resistor | 0.00080 | 0.00159 |
| SMT CHIP RESISTOR 130 KOhm,1%,0.063 CRCW0402130KFKTD | VISHAY INTERTECHNOLOGY IN | RESISTOR | 1 | 1.327 | 10 | 1.00 | §9 Resistor | 0.00088 | 0.00088 |
| SMT CHIP RESISTOR 150 KOhm,1%,0.063 CRCW0402150KFKTD | VISHAY RESISTIVE SYSTEMS | RESISTOR | 2 | 1.327 | 10 | 1.00 | §9 Resistor | 0.00088 | 0.00175 |
| SMT CHIP RESISTOR 69.8 KOhm,1%,0.06 CRCW040269K8FKTD | VISHAY DALE | RESISTOR | 1 | 1.327 | 10 | 1.00 | §9 Resistor | 0.00080 | 0.00080 |
| CHIP RESIS 80.6KOHM 1^ .063W T&R CRCW040280K6FKTD | VISHAY RESISTIVE SYSTEMS | IC | 2 | 0.374 | 14 | 1.00 | §5.1 Monolithic | 0.25900 | 0.51800 |
| SMT CHIP RESISTOR 20.5 KOhm,1%,0.06 CRCW040220K5FKTD | VISHAY DALE | RESISTOR | 2 | 1.327 | 10 | 1.00 | §9 Resistor | 0.00080 | 0.00159 |
| CHIP RESIS 44.2KOHM 1% .063W T&R CRCW040244K2FKTD | VISHAY RESISTIVE SYSTEMS | IC | 2 | 0.374 | 14 | 1.00 | §5.1 Monolithic | 0.25900 | 0.51800 |
| SMT CHIP RESISTOR 4.87 KOhm,1%,0.06 CRCW04024K87FKTD | VISHAY DALE | RESISTOR | 1 | 1.327 | 10 | 1.00 | §9 Resistor | 0.00080 | 0.00080 |
| CHIP RESIS 33.2 OHM 1% .063W T/R CRCW040233R2FKED | VISHAY RESISTIVE SYSTEMS | IC | 52 | 0.374 | 14 | 1.00 | §5.1 Monolithic | 0.25900 | 13.46805 |
| CHIP RESIS 18.0 KOHM 0.1% 0.063W T& TNPW040218K0BETD | VISHAY INTERTECHNOLOGY IN | IC | 4 | 0.374 | 14 | 1.00 | §5.1 Monolithic | 0.25900 | 1.03600 |
| CHIP RESIS 59.7 KOHM 0.1% 0.063W T& TNPW040259K7BETD | VISHAY INTERTECHNOLOGY IN | IC | 2 | 0.374 | 14 | 1.00 | §5.1 Monolithic | 0.25900 | 0.51800 |
| SMT CHIP RESISTOR 4.75 KOhm,1%,0.06 CRCW04024K75FKED | VISHAY RESISTIVE SYSTEMS | RESISTOR | 50 | 1.327 | 10 | 1.00 | §9 Resistor | 0.00080 | 0.03980 |
| SMT CHIP CAPACITOR 1000pF,10%,100V, C0402C102K1GALTU | KEMET ELECTRONICS SA | CAPACITOR | 1 | 1.327 | 10 | 1.00 | §10 Capacitor | 0.00146 | 0.00146 |
| Resistor 1.96 Kohm, 1%, 0.063W, T&R CRCW04021K96FKED | VISHAY DALE | RESISTOR | 1 | 1.327 | 10 | 1.00 | §9 Resistor | 0.00080 | 0.00080 |
| CHIP RESISTOR 71.5KOHM 1% .063W T&R CRCW040271K5FKED | VISHAY RESISTIVE SYSTEMS | RESISTOR | 1 | 1.327 | 10 | 1.00 | §9 Resistor | 0.00080 | 0.00080 |
| CHIP RESIS 11KOHM 1^ .063W T&R CRCW040211K0FKED | VISHAY DALE | IC | 2 | 0.374 | 14 | 1.00 | §5.1 Monolithic | 0.25900 | 0.51800 |
| SMT CHIP CAPACITOR HIGH-VOLTAGE 100 C1206C102KGRACTU | KEMET ELECTRONICS SA | CAPACITOR | 1 | 1.327 | 10 | 1.00 | §10 Capacitor | 0.00146 | 0.00146 |
| CHIP CAP 220PF 2% 50V COG T&R C0402C221G5GACTU | KMT | CAPACITOR | 24 | 1.327 | 10 | 1.00 | §10 Capacitor | 0.00111 | 0.02674 |
| SMT CHIP CAPACITOR 100pF,2%,100V,C0 C0402C101G1GACTU | KEMET ELECTRONICS SA | CAPACITOR | 4 | 1.327 | 10 | 1.00 | §10 Capacitor | 0.00097 | 0.00387 |
| CHIP CAP 4700PF 10% 100V X7R T&R C0603C472K1RACTU | KEMET | CAPACITOR | 4 | 1.327 | 10 | 1.00 | §10 Capacitor | 0.00193 | 0.00773 |
| SMT CHIP CAPACITOR 10uF,10%,25V,X7R C1206C106K3RACTU | KEMET ELECTRONICS SA | CAPACITOR | 34 | 1.327 | 10 | 1.00 | §10 Capacitor | 0.00768 | 0.26112 |
| 3.3V RS485 TRANSCEIVER WITH INTEGRA LTC2855HDE#PBF | LINEAR (Analog Devices) | IC | 6 | 0.374 | 14 | 1.00 | §5.1 Monolithic | 0.25900 | 1.55401 |
| SMT ULTRA LOW CAPACITANCE TRANSIENT RCLAMP0531T.TCT | SEMTECH CORPORATION | IC | 4 | 0.374 | 14 | 1.00 | §5.1 Monolithic | 0.25900 | 1.03600 |
| SMT TRANSFORMER 10/100BASE-TX SINGL 100B-1003XNL | INRCORE | INDUCTOR | 1 | 1.230 | 10 | 1.00 | §11 Inductor | 0.01230 | 0.01230 |
| CHIP CAP 0.1UF 10% 25VOLT 40X20 X7R 04023C104KAT2A | KYOCERA AVX Corporation | CAPACITOR | 46 | 1.327 | 10 | 1.00 | §10 Capacitor | 0.00335 | 0.15419 |
| LED BLUE 0402 SMD VLMB1500-GS08 | VISHAY SEMICONDUCTOR | DIODE | 2 | 3.739 | 19 | 1.00 | §7 Diode | 0.05273 | 0.10546 |
| CHIP CAP 0.1UF 10% 100V X7R T&R 06031C104K4T2A | AVX | CAPACITOR | 8 | 1.327 | 10 | 1.00 | §10 Capacitor | 0.00335 | 0.02682 |
| DIODE TVS , 440W, 5.5V Bidirectiona D5V0S1B2LP | DIODES INC | DIODE | 24 | 3.739 | 19 | 1.00 | §7 Diode | 0.05273 | 1.26554 |
| CHIP COMMON MODE CHOKE COIL 390OHM DLW21SH391XQ2L | MURATA POWER SOLUTIONS IN | IC | 12 | 0.374 | 14 | 1.00 | §5.1 Monolithic | 0.25900 | 3.10801 |
| IC DC/DC STEP DOWN REGULATOR 8A 20V LTM4657IY | ANALOG DEVICES | IC | 2 | 0.374 | 14 | 1.00 | §5.1 Monolithic | 0.12845 | 0.25689 |
| OSC 25.0 MHz -40 to +105 625L3G25M00000 | CTS | IC | 2 | 0.374 | 14 | 1.00 | §5.1 Monolithic | 0.25900 | 0.51800 |
| CHIP RESISTOR 34.8 Kohm 1% 0.063W T CRCW040234K8FKED | VISHAY DALE | RESISTOR | 1 | 1.327 | 10 | 1.00 | §9 Resistor | 0.00080 | 0.00080 |
| CHIP RESIS 1KOHM 1% .75W 1206 HP CRCW12061K00FKEAHP | VISHAY RESISTIVE SYSTEMS | IC | 8 | 0.374 | 14 | 1.00 | §5.1 Monolithic | 0.25900 | 2.07201 |
| CHIP EMI FILTER FERRITE BEAD 120 Oh BLM15PX121SN1D | MURATA MANUFACTURING CO. | IC | 29 | 0.374 | 14 | 1.00 | §5.1 Monolithic | 0.25900 | 7.51103 |
| SMT CHIP EMI FILTER FERRITE BEAD 22 BLM18KG221SN1D | MURATA MANUFACTURING CO. | IC | 6 | 0.374 | 14 | 1.00 | §5.1 Monolithic | 0.25900 | 1.55401 |
| SMT CHIP CAPACITOR1uF,10%,50V,X7R,T 06035C105KAT2A | KYOCERA AVX Corporation | CAPACITOR | 23 | 1.327 | 10 | 1.00 | §10 Capacitor | 0.00507 | 0.11670 |
| SINGLE 2 INPUT POSITIVE AND GATE SN74LVC1G08QDRYRQ1 | TEXAS | IC | 2 | 0.374 | 14 | 1.00 | §5.1 Monolithic | 0.25900 | 0.51800 |
| SMT HCMOS CRYSTAL TCXO 40 MHz,3.3V, 536EL400X2GT5 | CTS | CRYSTAL | 1 | 1.163 | 10 | 1.00 | §18 Crystal | 0.15115 | 0.15115 |
| SMT CHIP CAPACITOR 10uF,10%,10V,X7R GRM188Z71A106KA73D | MURATA MANUFACTURING CO. | CAPACITOR | 148 | 1.327 | 10 | 1.00 | §10 Capacitor | 0.00768 | 1.13664 |
| SMT 10/100 Mbps ETHERNET PHYSICAL L DP83822HRHBR | TEXAS INSTRUMENTS INC | IC | 1 | 0.374 | 14 | 1.00 | §5.1 Monolithic | 0.25900 | 0.25900 |
| SMT CHIP CAPACITOR 22uF,10%,10V,X7R C1206C226K8RACTU | KEMET ELECTRONICS SA | CAPACITOR | 44 | 1.327 | 10 | 1.00 | §10 Capacitor | 0.00885 | 0.38944 |
| SERIAL NOR FLASH 1GB 1.8V MT25QU01GBBB8E12-0AUT | MICRON TECHNOLOGY | IC | 1 | 0.374 | 14 | 1.00 | §5.1 Monolithic | 0.25900 | 0.25900 |
| CHIP CAP CER 1UF 10% 10V X7R 0402 T CC0402KRX7R6BB105 | YAGEO | CAPACITOR | 211 | 1.327 | 10 | 1.00 | §10 Capacitor | 0.00507 | 1.07061 |
| CHIP CAP 47UF 20^ 10V X7R T&R GRM32ER71A476ME15L | MURATA MANUFACTURING CO. | CAPACITOR | 1 | 1.327 | 10 | 1.00 | §10 Capacitor | 0.01015 | 0.01015 |
| CHIP RESIS 499 OHM 0.1^ 0.063W T&R TNPW0402499RBEED | VISHAY RESISTIVE SYSTEMS | IC | 1 | 0.374 | 14 | 1.00 | §5.1 Monolithic | 0.25900 | 0.25900 |
| TDA4VEN PROCESSOR 18X18MM 594FCBGA TDA4VEN8K5AAMWRQ1 | TEXAS | IC | 1 | 0.374 | 14 | 1.00 | §5.1 Monolithic | 4.34145 | 4.34145 |
| AGILEX5 E FPGA NO HPS 656K 23X23MM A5EC065BB23AI6X | TECNOLOGICA COMPONENTS EL | IC | 1 | 0.374 | 14 | 1.00 | §5.1 Monolithic | 5.21252 | 5.21252 |
| Connector PCI Express PCIe M2 Conne MDT275M01001 | AMPHENOL | CONNECTOR | 1 | — | 5.00 | 1.00 | §15 Connector | 1.77977 | 1.77977 |
| LPDDR4 SDRAM 2GB(16Gb) AUTOMOTIVE U MT53E512M32D1ZW-046AUT:B | MICRON TECHNOLOGY | IC | 1 | 0.374 | 14 | 1.00 | §5.1 Monolithic | 0.25900 | 0.25900 |
| DC/DC 20VIN 20A STEP-DOWN REGULATOR LTM4640IY#PBF | ANALOG DEVICES | IC | 2 | 0.374 | 14 | 1.00 | §5.1 Monolithic | 0.12845 | 0.25689 |
| 16VIN 4A SILENT SWITCHER REGULATOR LTM4732IY#PBF | ANALOG DEVICES | IC | 7 | 0.374 | 14 | 1.00 | §5.1 Monolithic | 0.12845 | 0.89911 |
| 20V 1A ULTRA LOW NOISE-PSRR LINEAR LT3041ADE#PBF | ANALOG DEVICES | IC | 3 | 0.374 | 14 | 1.00 | §5.1 Monolithic | 0.12845 | 0.38533 |
| NOR FLASH OSPI 2Gbit 1.8V AUTOMOTIV MT35XU02GCBA1G12-A0AUT | MICRON TECHNOLOGY | IC | 1 | 0.374 | 14 | 1.00 | §5.1 Monolithic | 0.25900 | 0.25900 |
| ETHERNET SWITCH 5 PORTS WITH 4X 100 LAN9370-V/KCXVAO | MICROCHIP | IC | 1 | 0.374 | 14 | 1.00 | §5.1 Monolithic | 0.25900 | 0.25900 |
| HCMOS CLOCK OSCILLATOR 1.8V 25MHZ 5 625M3G025M00000 | CTS CORPORATION | CRYSTAL | 4 | 1.163 | 10 | 1.00 | §18 Crystal | 0.15115 | 0.60459 |
| LPDDR4 SDRAM 4GB(32Gb) AUTOMOTIVE T MT53E1G32D2FW-046AAT:C | MICRON TECHNOLOGY | IC | 1 | 0.374 | 14 | 1.00 | §5.1 Monolithic | 0.25900 | 0.25900 |
| CAP EMI FILTER 3TERM 1UF 4V 0402 20 NFM15HC105D0G3D | MURATA | CAPACITOR | 42 | 1.327 | 10 | 1.00 | §10 Capacitor | 0.00507 | 0.21311 |
| SINGLE GMSL2 TO MIPI CSI-2 DESERIAL MAX96714GTJ/VY+T | ANALOG DEVICES | IC | 2 | 0.374 | 14 | 1.00 | §5.1 Monolithic | 0.25900 | 0.51800 |
| FILTER EMI MLC 10UF 20% 4V 2A 0603 NFM18HC106D0G3L | MURATA | IC | 10 | 0.374 | 14 | 1.00 | §5.1 Monolithic | 0.25900 | 2.59001 |
| SMT MICRO SOCKET 1.27MMX2.54MM 20PI ASP-248396-02 | SAMTEC | IC | 1 | 0.374 | 14 | 1.00 | §5.1 Monolithic | 4.34145 | 4.34145 |
| Connector to EOS SEAF-20-01-L-04-2-RA-GP-T | SAMTEC | CONNECTOR | 1 | — | 5.00 | 1.00 | §15 Connector | 1.77977 | 1.77977 |
| Connector to DLT G125-MS12605M2P | HARWIN | CONNECTOR | 1 | — | 5.00 | 1.00 | §15 Connector | 1.77977 | 1.77977 |
| Connector to UMBILI G125-MS13405M2P | HARWIN | CONNECTOR | 1 | — | 5.00 | 1.00 | §15 Connector | 1.77977 | 1.77977 |
| Connector Board to Board 40P 2X20 0 QTE-020-05-L-D-A-K | SAMTEC | CONNECTOR | 1 | — | 5.00 | 1.00 | §15 Connector | 1.77977 | 1.77977 |
| CHIP RESIS 6.49 KOHM 0.1% 0.063W 04 TNPW04026K49BEED | VISHAY INTERTECHNOLOGY IN | IC | 2 | 0.374 | 14 | 1.00 | §5.1 Monolithic | 0.25900 | 0.51800 |
| 1210 COMMON MODE CHOKE COIL 200UH 7 DLW32MH201XK2L | MURATA MANUFACTURING CO. | IC | 4 | 0.374 | 14 | 1.00 | §5.1 Monolithic | 0.25900 | 1.03600 |
| FRAM 2MBIT SPI 50MHZ 8X256K 1.8V DF CY15V102QN-50LHXI | INFINEON TECHNOLOGIES AG | IC | 1 | 0.374 | 14 | 1.00 | §5.1 Monolithic | 0.25900 | 0.25900 |
| ADJUSTABLE SEQUENCING/SUPERVISORY C MAX16895AALT+T | ANALOG DEVICES | IC | 1 | 0.374 | 14 | 1.00 | §5.1 Monolithic | 0.25900 | 0.25900 |
| CHIP RESIS 240 OHM 1% .063W 0402 T& CRCW0402240RFKED | VISHAY RESISTIVE SYSTEMS | IC | 7 | 0.374 | 14 | 1.00 | §5.1 Monolithic | 0.25900 | 1.81301 |
| SMT CHIP RESISTOR 12 KOhm,1%,0.063 CRCW040212K0FKTD | VISHAY DALE | RESISTOR | 1 | 1.327 | 10 | 1.00 | §9 Resistor | 0.00080 | 0.00080 |
| SMT CHIP CAPACITOR 2.2uF,10%,10V,X7 C0603C225K8RACTU | KEMET ELECTRONICS SA | CAPACITOR | 4 | 1.327 | 10 | 1.00 | §10 Capacitor | 0.00585 | 0.02339 |
| CAP CHIP 0.01UF 10% 25V X7R 0201 T& GRM033R71E103KE14D | MURATA MANUFACTURING CO. | CAPACITOR | 10 | 1.327 | 10 | 1.00 | §10 Capacitor | 0.00221 | 0.02215 |
| RES SMD 100K OHM 1% 0.33W 0603 CRCW0603100KFKEAHP | VISHAY SEMICONDUCTOR | RESISTOR | 4 | 1.327 | 10 | 1.00 | §9 Resistor | 0.00080 | 0.00318 |
| nan nan | nan | IC | 1 | 0.374 | 14 | 1.00 | §5.1 Monolithic | 0.25900 | 0.25900 |
| nan nan | nan | IC | 1 | 0.374 | 14 | 1.00 | §5.1 Monolithic | 0.25900 | 0.25900 |
| סה"כ מערכת |  |  |  |  |  |  |  |  | 76.14182 |

סה"כ λ_sys (MIL-217F) = 76.1418 כשלים/10⁶ שע  (76141.8 FIT)  →  MTBF = 13,133 שעות  (1.5 שנים)

# 6  תוצאות – ניתוח נתוני יצרנים

טמפרטורת הפעלה: 40.0°C | תיקון טמפרטורה: Arrhenius

| תיאור / Part# | יצרן | קטגוריה | כמ' | FIT@25°C | FIT@T | Ea [eV] | מקור נתונים | λ_p יחידה (כשלים/10⁶שע) | λ_p סה"כ (כשלים/10⁶שע) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SMT CHIP JUMPER RESISTOR 0 Ohm,0.06 CRCW04020000ZSTD | VISHAY RESISTIVE SYSTEMS | RESISTOR_FILM | 41 | 4.0 | 5.3 | — | Vishay CRCW Series – Thick-Film SMD Resi | 0.00529 | 0.21693 |
| SMT CHIP RESISTOR 1 KOhm,1%,0.063W, CRCW04021K00FKTD | VISHAY RESISTIVE SYSTEMS | RESISTOR_FILM | 26 | 4.0 | 5.3 | — | Vishay CRCW Series – Thick-Film SMD Resi | 0.00529 | 0.13757 |
| SMT CHIP RESISTOR 75 Ohm,1%,0.25W,T CRCW120675R0FKTA | VISHAY SEMICONDUCTOR | RESISTOR_POWER | 2 | 8.0 | 11.6 | — | Vishay Dale – Power Film Resistor Reliab | 0.01162 | 0.02323 |
| SMT CHIP RESISTOR 100 KOhm,1%,0.063 CRCW0402100KFKTD | VISHAY RESISTIVE SYSTEMS | RESISTOR_FILM | 20 | 4.0 | 5.3 | — | Vishay CRCW Series – Thick-Film SMD Resi | 0.00529 | 0.10582 |
| SMT CHIP RESISTOR 49.9 Ohm,1%,0.063 CRCW040249R9FKTD | VISHAY RESISTIVE SYSTEMS | RESISTOR_FILM | 4 | 4.0 | 5.3 | — | Vishay CRCW Series – Thick-Film SMD Resi | 0.00529 | 0.02116 |
| SMT CHIP RESISTOR 200 KOhm,1%,0.063 CRCW0402200KFKTD | VISHAY RESISTIVE SYSTEMS | RESISTOR_FILM | 1 | 4.0 | 5.3 | — | Vishay CRCW Series – Thick-Film SMD Resi | 0.00529 | 0.00529 |
| SMT CHIP RESISTOR 24.9 KOhm,1%,0.06 CRCW040224K9FKTD | VISHAY RESISTIVE SYSTEMS | RESISTOR_FILM | 1 | 4.0 | 5.3 | — | Vishay CRCW Series – Thick-Film SMD Resi | 0.00529 | 0.00529 |
| SMT CHIP RESISTOR 48.7 KOhm,1%,0.06 CRCW040248K7FKTD | VISHAY INTERTECHNOLOGY IN | RESISTOR_FILM | 2 | 4.0 | 5.3 | — | Vishay CRCW Series – Thick-Film SMD Resi | 0.00529 | 0.01058 |
| CHIP RESIS 56.2KOHM 1% .063W T&R CRCW040256K2FKTD | VISHAY RESISTIVE SYSTEMS | RESISTOR_FILM | 2 | 4.0 | 5.3 | — | Vishay CRCW Series – Thick-Film SMD Resi | 0.00529 | 0.01058 |
| SMT CHIP RESISTOR 402 Ohm,1%,0.063W CRCW0402402RFKTD | VISHAY DALE | RESISTOR_FILM | 2 | 4.0 | 5.3 | — | Vishay CRCW Series – Thick-Film SMD Resi | 0.00529 | 0.01058 |
| SMT CHIP RESISTOR 40.2 KOhm,1%,0.06 CRCW040240K2FKTD | VISHAY DALE | RESISTOR_FILM | 2 | 4.0 | 5.3 | — | Vishay CRCW Series – Thick-Film SMD Resi | 0.00529 | 0.01058 |
| CHIP RESIS 150 OHM 1% .063W T&R CRCW0402150RFKTD | VISHAY RESISTIVE SYSTEMS | RESISTOR_FILM | 3 | 4.0 | 5.3 | — | Vishay CRCW Series – Thick-Film SMD Resi | 0.00529 | 0.01587 |
| SMT CHIP RESISTOR 2.21 KOhm,1%,0.06 CRCW04022K21FKTD | VISHAY RESISTIVE SYSTEMS | RESISTOR_FILM | 2 | 4.0 | 5.3 | — | Vishay CRCW Series – Thick-Film SMD Resi | 0.00529 | 0.01058 |
| SMT CHIP RESISTOR 4.7 KOhm,1%,0.063 CRCW04024K70FKTD | VISHAY INTERTECHNOLOGY IN | RESISTOR_FILM | 3 | 4.0 | 5.3 | — | Vishay CRCW Series – Thick-Film SMD Resi | 0.00529 | 0.01587 |
| SMT CHIP RESISTOR 137 KOhm,1%,0.063 CRCW0402137KFKTD | VISHAY INTERTECHNOLOGY IN | RESISTOR_FILM | 6 | 4.0 | 5.3 | — | Vishay CRCW Series – Thick-Film SMD Resi | 0.00529 | 0.03175 |
| SMT CHIP CAPACITOR 10000pF,10%,50V, C0402C103K5RALTU | KEMET ELECTRONICS SA | CAPACITOR_CERAMIC | 6 | 1.5 | 2.9 | — | KEMET / Murata / YAGEO – MLCC X7R/C0G Re | 0.00288 | 0.01729 |
| CHIP CAP 0.1UF 10% 16V X7R T&R GRM033Z71C104KE14D | MURATA | CAPACITOR_CERAMIC | 199 | 1.5 | 2.9 | — | KEMET / Murata / YAGEO – MLCC X7R/C0G Re | 0.00288 | 0.57332 |
| SMT CHIP RESISTOR 10 KOhm,1%,0.063W CRCW040210K0FKED | VISHAY RESISTIVE SYSTEMS | RESISTOR_FILM | 51 | 4.0 | 5.3 | — | Vishay CRCW Series – Thick-Film SMD Resi | 0.00529 | 0.26984 |
| SMT CHIP RESISTOR 2 KOhm,1%,0.063W, CRCW04022K00FKED | VISHAY RESISTIVE SYSTEMS | RESISTOR_FILM | 1 | 4.0 | 5.3 | — | Vishay CRCW Series – Thick-Film SMD Resi | 0.00529 | 0.00529 |
| SMT CHIP RESISTOR 499 Ohm,1%,0.063W CRCW0402499RFKED | VISHAY RESISTIVE SYSTEMS | RESISTOR_FILM | 7 | 4.0 | 5.3 | — | Vishay CRCW Series – Thick-Film SMD Resi | 0.00529 | 0.03704 |
| SMT CHIP RESISTOR 49.9 KOhm,1%,0.06 CRCW040249K9FKED | VISHAY RESISTIVE SYSTEMS | RESISTOR_FILM | 1 | 4.0 | 5.3 | — | Vishay CRCW Series – Thick-Film SMD Resi | 0.00529 | 0.00529 |
| SMT CHIP RESISTOR 42.2 KOhm,1%,0.06 CRCW040242K2FKED | VISHAY RESISTIVE SYSTEMS | RESISTOR_FILM | 2 | 4.0 | 5.3 | — | Vishay CRCW Series – Thick-Film SMD Resi | 0.00529 | 0.01058 |
| SMT CHIP RESISTOR 6.19 KOhm,1%,0.06 CRCW04026K19FKED | VISHAY INTERTECHNOLOGY IN | RESISTOR_FILM | 1 | 4.0 | 5.3 | — | Vishay CRCW Series – Thick-Film SMD Resi | 0.00529 | 0.00529 |
| Resistor SMT, 3.01Kohm, 1%, 0.063W, CRCW04023K01FKED | VISHAY RESISTIVE SYSTEMS | RESISTOR_FILM | 2 | 4.0 | 5.3 | — | Vishay CRCW Series – Thick-Film SMD Resi | 0.00529 | 0.01058 |
| SMT CHIP RESISTOR 23.2 Kohm,1%,0.06 CRCW040223K2FKTD | VISHAY DALE | RESISTOR_FILM | 3 | 4.0 | 5.3 | — | Vishay CRCW Series – Thick-Film SMD Resi | 0.00529 | 0.01587 |
| SMT CHIP RESISTOR 8.45 KOhm,1%,0.06 CRCW04028K45FKTD | VISHAY DALE | RESISTOR_FILM | 1 | 4.0 | 5.3 | — | Vishay CRCW Series – Thick-Film SMD Resi | 0.00529 | 0.00529 |
| SMT CHIP RESISTOR 10.7 Kohm,1%,0.06 CRCW040210K7FKTD | VISHAY DALE | RESISTOR_FILM | 2 | 4.0 | 5.3 | — | Vishay CRCW Series – Thick-Film SMD Resi | 0.00529 | 0.01058 |
| SMT CHIP RESISTOR 130 KOhm,1%,0.063 CRCW0402130KFKTD | VISHAY INTERTECHNOLOGY IN | RESISTOR_FILM | 1 | 4.0 | 5.3 | — | Vishay CRCW Series – Thick-Film SMD Resi | 0.00529 | 0.00529 |
| SMT CHIP RESISTOR 150 KOhm,1%,0.063 CRCW0402150KFKTD | VISHAY RESISTIVE SYSTEMS | RESISTOR_FILM | 2 | 4.0 | 5.3 | — | Vishay CRCW Series – Thick-Film SMD Resi | 0.00529 | 0.01058 |
| SMT CHIP RESISTOR 69.8 KOhm,1%,0.06 CRCW040269K8FKTD | VISHAY DALE | RESISTOR_FILM | 1 | 4.0 | 5.3 | — | Vishay CRCW Series – Thick-Film SMD Resi | 0.00529 | 0.00529 |
| CHIP RESIS 80.6KOHM 1^ .063W T&R CRCW040280K6FKTD | VISHAY RESISTIVE SYSTEMS | RESISTOR_FILM | 2 | 4.0 | 5.3 | — | Vishay CRCW Series – Thick-Film SMD Resi | 0.00529 | 0.01058 |
| SMT CHIP RESISTOR 20.5 KOhm,1%,0.06 CRCW040220K5FKTD | VISHAY DALE | RESISTOR_FILM | 2 | 4.0 | 5.3 | — | Vishay CRCW Series – Thick-Film SMD Resi | 0.00529 | 0.01058 |
| CHIP RESIS 44.2KOHM 1% .063W T&R CRCW040244K2FKTD | VISHAY RESISTIVE SYSTEMS | RESISTOR_FILM | 2 | 4.0 | 5.3 | — | Vishay CRCW Series – Thick-Film SMD Resi | 0.00529 | 0.01058 |
| SMT CHIP RESISTOR 4.87 KOhm,1%,0.06 CRCW04024K87FKTD | VISHAY DALE | RESISTOR_FILM | 1 | 4.0 | 5.3 | — | Vishay CRCW Series – Thick-Film SMD Resi | 0.00529 | 0.00529 |
| CHIP RESIS 33.2 OHM 1% .063W T/R CRCW040233R2FKED | VISHAY RESISTIVE SYSTEMS | RESISTOR_FILM | 52 | 4.0 | 5.3 | — | Vishay CRCW Series – Thick-Film SMD Resi | 0.00529 | 0.27513 |
| CHIP RESIS 18.0 KOHM 0.1% 0.063W T& TNPW040218K0BETD | VISHAY INTERTECHNOLOGY IN | RESISTOR_PRECISION | 4 | 3.0 | 4.0 | — | Vishay TNPW Series – Precision Thin-Film | 0.00397 | 0.01587 |
| CHIP RESIS 59.7 KOHM 0.1% 0.063W T& TNPW040259K7BETD | VISHAY INTERTECHNOLOGY IN | RESISTOR_PRECISION | 2 | 3.0 | 4.0 | — | Vishay TNPW Series – Precision Thin-Film | 0.00397 | 0.00794 |
| SMT CHIP RESISTOR 4.75 KOhm,1%,0.06 CRCW04024K75FKED | VISHAY RESISTIVE SYSTEMS | RESISTOR_FILM | 50 | 4.0 | 5.3 | — | Vishay CRCW Series – Thick-Film SMD Resi | 0.00529 | 0.26455 |
| SMT CHIP CAPACITOR 1000pF,10%,100V, C0402C102K1GALTU | KEMET ELECTRONICS SA | CAPACITOR_CERAMIC | 1 | 1.5 | 2.9 | — | KEMET / Murata / YAGEO – MLCC X7R/C0G Re | 0.00288 | 0.00288 |
| Resistor 1.96 Kohm, 1%, 0.063W, T&R CRCW04021K96FKED | VISHAY DALE | RESISTOR_FILM | 1 | 4.0 | 5.3 | — | Vishay CRCW Series – Thick-Film SMD Resi | 0.00529 | 0.00529 |
| CHIP RESISTOR 71.5KOHM 1% .063W T&R CRCW040271K5FKED | VISHAY RESISTIVE SYSTEMS | RESISTOR_FILM | 1 | 4.0 | 5.3 | — | Vishay CRCW Series – Thick-Film SMD Resi | 0.00529 | 0.00529 |
| CHIP RESIS 11KOHM 1^ .063W T&R CRCW040211K0FKED | VISHAY DALE | RESISTOR_FILM | 2 | 4.0 | 5.3 | — | Vishay CRCW Series – Thick-Film SMD Resi | 0.00529 | 0.01058 |
| SMT CHIP CAPACITOR HIGH-VOLTAGE 100 C1206C102KGRACTU | KEMET ELECTRONICS SA | CAPACITOR_CERAMIC_HI | 1 | 3.0 | 5.8 | — | KEMET – High-Voltage MLCC Reliability Re | 0.00576 | 0.00576 |
| CHIP CAP 220PF 2% 50V COG T&R C0402C221G5GACTU | KMT | CAPACITOR_CERAMIC | 24 | 1.5 | 2.9 | — | KEMET / Murata / YAGEO – MLCC X7R/C0G Re | 0.00288 | 0.06914 |
| SMT CHIP CAPACITOR 100pF,2%,100V,C0 C0402C101G1GACTU | KEMET ELECTRONICS SA | CAPACITOR_CERAMIC | 4 | 1.5 | 2.9 | — | KEMET / Murata / YAGEO – MLCC X7R/C0G Re | 0.00288 | 0.01152 |
| CHIP CAP 4700PF 10% 100V X7R T&R C0603C472K1RACTU | KEMET | CAPACITOR_CERAMIC | 4 | 1.5 | 2.9 | — | KEMET / Murata / YAGEO – MLCC X7R/C0G Re | 0.00288 | 0.01152 |
| SMT CHIP CAPACITOR 10uF,10%,25V,X7R C1206C106K3RACTU | KEMET ELECTRONICS SA | CAPACITOR_CERAMIC | 34 | 1.5 | 2.9 | — | KEMET / Murata / YAGEO – MLCC X7R/C0G Re | 0.00288 | 0.09795 |
| 3.3V RS485 TRANSCEIVER WITH INTEGRA LTC2855HDE#PBF | LINEAR (Analog Devices) | IC_TRANSCEIVER | 6 | 70.0 | 258.2 | — | Analog Devices LTC2855 / TI – Transceive | 0.25817 | 1.54900 |
| SMT ULTRA LOW CAPACITANCE TRANSIENT RCLAMP0531T.TCT | SEMTECH CORPORATION | DIODE_TVS_BIDIR | 4 | 12.0 | 25.3 | — | Vishay / Semtech / Diodes Inc – TVS Diod | 0.02530 | 0.10119 |
| SMT TRANSFORMER 10/100BASE-TX SINGL 100B-1003XNL | INRCORE | TRANSFORMER | 1 | 55.0 | 96.2 | — | Industry typical – pulse transformer / L | 0.09622 | 0.09622 |
| CHIP CAP 0.1UF 10% 25VOLT 40X20 X7R 04023C104KAT2A | KYOCERA AVX Corporation | CAPACITOR_CERAMIC | 46 | 1.5 | 2.9 | — | KEMET / Murata / YAGEO – MLCC X7R/C0G Re | 0.00288 | 0.13253 |
| LED BLUE 0402 SMD VLMB1500-GS08 | VISHAY SEMICONDUCTOR | LED | 2 | 28.0 | 71.1 | — | Vishay VLMB Series – SMD LED Reliability | 0.07112 | 0.14225 |
| CHIP CAP 0.1UF 10% 100V X7R T&R 06031C104K4T2A | AVX | CAPACITOR_CERAMIC | 8 | 1.5 | 2.9 | — | KEMET / Murata / YAGEO – MLCC X7R/C0G Re | 0.00288 | 0.02305 |
| DIODE TVS , 440W, 5.5V Bidirectiona D5V0S1B2LP | DIODES INC | DIODE_TVS_BIDIR | 24 | 12.0 | 25.3 | — | Vishay / Semtech / Diodes Inc – TVS Diod | 0.02530 | 0.60713 |
| CHIP COMMON MODE CHOKE COIL 390OHM DLW21SH391XQ2L | MURATA POWER SOLUTIONS IN | CHOKE_COMMON_MODE | 12 | 8.0 | 11.6 | — | Murata DLW Series – Common-Mode Choke Re | 0.01162 | 0.13938 |
| IC DC/DC STEP DOWN REGULATOR 8A 20V LTM4657IY | ANALOG DEVICES | IC_DCDC_MODULE | 2 | 280.0 | 1032.7 | — | Analog Devices LTM Series – µModule DC-D | 1.03266 | 2.06533 |
| OSC 25.0 MHz -40 to +105 625L3G25M00000 | CTS | OSCILLATOR_HCMOS | 2 | 90.0 | 189.7 | — | CTS Corporation – HCMOS Oscillator Relia | 0.18973 | 0.37945 |
| CHIP RESISTOR 34.8 Kohm 1% 0.063W T CRCW040234K8FKED | VISHAY DALE | RESISTOR_FILM | 1 | 4.0 | 5.3 | — | Vishay CRCW Series – Thick-Film SMD Resi | 0.00529 | 0.00529 |
| CHIP RESIS 1KOHM 1% .75W 1206 HP CRCW12061K00FKEAHP | VISHAY RESISTIVE SYSTEMS | RESISTOR_FILM | 8 | 4.0 | 5.3 | — | Vishay CRCW Series – Thick-Film SMD Resi | 0.00529 | 0.04233 |
| CHIP EMI FILTER FERRITE BEAD 120 Oh BLM15PX121SN1D | MURATA MANUFACTURING CO. | FERRITE_BEAD | 29 | 4.0 | 5.8 | — | Murata BLM Series – Ferrite Bead Reliabi | 0.00581 | 0.16843 |
| SMT CHIP EMI FILTER FERRITE BEAD 22 BLM18KG221SN1D | MURATA MANUFACTURING CO. | FERRITE_BEAD | 6 | 4.0 | 5.8 | — | Murata BLM Series – Ferrite Bead Reliabi | 0.00581 | 0.03485 |
| SMT CHIP CAPACITOR1uF,10%,50V,X7R,T 06035C105KAT2A | KYOCERA AVX Corporation | CAPACITOR_CERAMIC | 23 | 1.5 | 2.9 | — | KEMET / Murata / YAGEO – MLCC X7R/C0G Re | 0.00288 | 0.06626 |
| SINGLE 2 INPUT POSITIVE AND GATE SN74LVC1G08QDRYRQ1 | TEXAS | IC_LOGIC_GATE | 2 | 20.0 | 73.8 | — | TI SN74 Series – Logic Gate Reliability | 0.07376 | 0.14752 |
| SMT HCMOS CRYSTAL TCXO 40 MHz,3.3V, 536EL400X2GT5 | CTS | TCXO | 1 | 170.0 | 358.4 | — | CTS Corporation – TCXO Reliability Data | 0.35837 | 0.35837 |
| SMT CHIP CAPACITOR 10uF,10%,10V,X7R GRM188Z71A106KA73D | MURATA MANUFACTURING CO. | CAPACITOR_CERAMIC | 148 | 1.5 | 2.9 | — | KEMET / Murata / YAGEO – MLCC X7R/C0G Re | 0.00288 | 0.42639 |
| SMT 10/100 Mbps ETHERNET PHYSICAL L DP83822HRHBR | TEXAS INSTRUMENTS INC | IC_DIGITAL_COMPLEX | 1 | 130.0 | 479.5 | — | TI / Microchip – Digital IC Reliability | 0.47945 | 0.47945 |
| SMT CHIP CAPACITOR 22uF,10%,10V,X7R C1206C226K8RACTU | KEMET ELECTRONICS SA | CAPACITOR_CERAMIC | 44 | 1.5 | 2.9 | — | KEMET / Murata / YAGEO – MLCC X7R/C0G Re | 0.00288 | 0.12676 |
| SERIAL NOR FLASH 1GB 1.8V MT25QU01GBBB8E12-0AUT | MICRON TECHNOLOGY | IC_FLASH_NOR | 1 | 75.0 | 276.6 | — | Micron Technology – NOR/OSPI Flash Relia | 0.27661 | 0.27661 |
| CHIP CAP CER 1UF 10% 10V X7R 0402 T CC0402KRX7R6BB105 | YAGEO | CAPACITOR_CERAMIC | 211 | 1.5 | 2.9 | — | KEMET / Murata / YAGEO – MLCC X7R/C0G Re | 0.00288 | 0.60789 |
| CHIP CAP 47UF 20^ 10V X7R T&R GRM32ER71A476ME15L | MURATA MANUFACTURING CO. | CAPACITOR_CERAMIC | 1 | 1.5 | 2.9 | — | KEMET / Murata / YAGEO – MLCC X7R/C0G Re | 0.00288 | 0.00288 |
| CHIP RESIS 499 OHM 0.1^ 0.063W T&R TNPW0402499RBEED | VISHAY RESISTIVE SYSTEMS | RESISTOR_FILM | 1 | 4.0 | 5.3 | — | Vishay CRCW Series – Thick-Film SMD Resi | 0.00529 | 0.00529 |
| TDA4VEN PROCESSOR 18X18MM 594FCBGA TDA4VEN8K5AAMWRQ1 | TEXAS | IC_PROCESSOR | 1 | 450.0 | 1659.6 | — | TI TDA4 Series – Automotive SoC Reliabil | 1.65964 | 1.65964 |
| AGILEX5 E FPGA NO HPS 656K 23X23MM A5EC065BB23AI6X | TECNOLOGICA COMPONENTS EL | IC_FPGA | 1 | 900.0 | 3319.3 | — | Intel Agilex5 – FPGA Reliability Qualifi | 3.31927 | 3.31927 |
| Connector PCI Express PCIe M2 Conne MDT275M01001 | AMPHENOL | CONNECTOR | 1 | 28.0 | 37.0 | — | Samtec / Harwin / Amphenol – Connector R | 0.03703 | 0.03703 |
| LPDDR4 SDRAM 2GB(16Gb) AUTOMOTIVE U MT53E512M32D1ZW-046AUT:B | MICRON TECHNOLOGY | IC_DRAM_LPDDR4 | 1 | 90.0 | 331.9 | — | Micron Technology – LPDDR4 Reliability R | 0.33193 | 0.33193 |
| DC/DC 20VIN 20A STEP-DOWN REGULATOR LTM4640IY#PBF | ANALOG DEVICES | IC_DCDC_MODULE | 2 | 280.0 | 1032.7 | — | Analog Devices LTM Series – µModule DC-D | 1.03266 | 2.06533 |
| 16VIN 4A SILENT SWITCHER REGULATOR LTM4732IY#PBF | ANALOG DEVICES | IC_DCDC_MODULE | 7 | 280.0 | 1032.7 | — | Analog Devices LTM Series – µModule DC-D | 1.03266 | 7.22864 |
| 20V 1A ULTRA LOW NOISE-PSRR LINEAR LT3041ADE#PBF | ANALOG DEVICES | IC_LINEAR_REG | 3 | 55.0 | 202.8 | — | Analog Devices LT30xx Series – LDO Relia | 0.20284 | 0.60854 |
| NOR FLASH OSPI 2Gbit 1.8V AUTOMOTIV MT35XU02GCBA1G12-A0AUT | MICRON TECHNOLOGY | IC_FLASH_NOR | 1 | 75.0 | 276.6 | — | Micron Technology – NOR/OSPI Flash Relia | 0.27661 | 0.27661 |
| ETHERNET SWITCH 5 PORTS WITH 4X 100 LAN9370-V/KCXVAO | MICROCHIP | IC_DIGITAL_COMPLEX | 1 | 130.0 | 479.5 | — | TI / Microchip – Digital IC Reliability | 0.47945 | 0.47945 |
| HCMOS CLOCK OSCILLATOR 1.8V 25MHZ 5 625M3G025M00000 | CTS CORPORATION | OSCILLATOR_HCMOS | 4 | 90.0 | 189.7 | — | CTS Corporation – HCMOS Oscillator Relia | 0.18973 | 0.75891 |
| LPDDR4 SDRAM 4GB(32Gb) AUTOMOTIVE T MT53E1G32D2FW-046AAT:C | MICRON TECHNOLOGY | IC_DRAM_LPDDR4 | 1 | 90.0 | 331.9 | — | Micron Technology – LPDDR4 Reliability R | 0.33193 | 0.33193 |
| CAP EMI FILTER 3TERM 1UF 4V 0402 20 NFM15HC105D0G3D | MURATA | FERRITE_BEAD | 42 | 4.0 | 5.8 | — | Murata BLM Series – Ferrite Bead Reliabi | 0.00581 | 0.24394 |
| SINGLE GMSL2 TO MIPI CSI-2 DESERIAL MAX96714GTJ/VY+T | ANALOG DEVICES | IC_DESERIALIZER | 2 | 200.0 | 737.6 | — | Analog Devices MAX96714 – GMSL2 Deserial | 0.73762 | 1.47523 |
| FILTER EMI MLC 10UF 20% 4V 2A 0603 NFM18HC106D0G3L | MURATA | EMI_FILTER_CAP | 10 | 1.8 | 3.5 | — | Murata NFM Series – EMI Filter Reliabili | 0.00346 | 0.03457 |
| SMT MICRO SOCKET 1.27MMX2.54MM 20PI ASP-248396-02 | SAMTEC | CONNECTOR | 1 | 28.0 | 37.0 | — | Samtec / Harwin / Amphenol – Connector R | 0.03703 | 0.03703 |
| Connector to EOS SEAF-20-01-L-04-2-RA-GP-T | SAMTEC | CONNECTOR | 1 | 28.0 | 37.0 | — | Samtec / Harwin / Amphenol – Connector R | 0.03703 | 0.03703 |
| Connector to DLT G125-MS12605M2P | HARWIN | CONNECTOR | 1 | 28.0 | 37.0 | — | Samtec / Harwin / Amphenol – Connector R | 0.03703 | 0.03703 |
| Connector to UMBILI G125-MS13405M2P | HARWIN | CONNECTOR | 1 | 28.0 | 37.0 | — | Samtec / Harwin / Amphenol – Connector R | 0.03703 | 0.03703 |
| Connector Board to Board 40P 2X20 0 QTE-020-05-L-D-A-K | SAMTEC | CONNECTOR | 1 | 28.0 | 37.0 | — | Samtec / Harwin / Amphenol – Connector R | 0.03703 | 0.03703 |
| CHIP RESIS 6.49 KOHM 0.1% 0.063W 04 TNPW04026K49BEED | VISHAY INTERTECHNOLOGY IN | RESISTOR_PRECISION | 2 | 3.0 | 4.0 | — | Vishay TNPW Series – Precision Thin-Film | 0.00397 | 0.00794 |
| 1210 COMMON MODE CHOKE COIL 200UH 7 DLW32MH201XK2L | MURATA MANUFACTURING CO. | CHOKE_COMMON_MODE | 4 | 8.0 | 11.6 | — | Murata DLW Series – Common-Mode Choke Re | 0.01162 | 0.04646 |
| FRAM 2MBIT SPI 50MHZ 8X256K 1.8V DF CY15V102QN-50LHXI | INFINEON TECHNOLOGIES AG | IC_FRAM | 1 | 40.0 | 147.5 | — | Infineon CY15 Series – FRAM Reliability | 0.14752 | 0.14752 |
| ADJUSTABLE SEQUENCING/SUPERVISORY C MAX16895AALT+T | ANALOG DEVICES | IC_SUPERVISOR | 1 | 45.0 | 166.0 | — | Analog Devices MAX – Supervisor/Sequence | 0.16596 | 0.16596 |
| CHIP RESIS 240 OHM 1% .063W 0402 T& CRCW0402240RFKED | VISHAY RESISTIVE SYSTEMS | RESISTOR_FILM | 7 | 4.0 | 5.3 | — | Vishay CRCW Series – Thick-Film SMD Resi | 0.00529 | 0.03704 |
| SMT CHIP RESISTOR 12 KOhm,1%,0.063 CRCW040212K0FKTD | VISHAY DALE | RESISTOR_FILM | 1 | 4.0 | 5.3 | — | Vishay CRCW Series – Thick-Film SMD Resi | 0.00529 | 0.00529 |
| SMT CHIP CAPACITOR 2.2uF,10%,10V,X7 C0603C225K8RACTU | KEMET ELECTRONICS SA | CAPACITOR_CERAMIC | 4 | 1.5 | 2.9 | — | KEMET / Murata / YAGEO – MLCC X7R/C0G Re | 0.00288 | 0.01152 |
| CAP CHIP 0.01UF 10% 25V X7R 0201 T& GRM033R71E103KE14D | MURATA MANUFACTURING CO. | CAPACITOR_CERAMIC | 10 | 1.5 | 2.9 | — | KEMET / Murata / YAGEO – MLCC X7R/C0G Re | 0.00288 | 0.02881 |
| RES SMD 100K OHM 1% 0.33W 0603 CRCW0603100KFKEAHP | VISHAY SEMICONDUCTOR | RESISTOR_POWER | 4 | 8.0 | 11.6 | — | Vishay Dale – Power Film Resistor Reliab | 0.01162 | 0.04646 |
| nan nan | nan | IC_DIGITAL_COMPLEX | 1 | 130.0 | 479.5 | — | TI / Microchip – Digital IC Reliability | 0.47945 | 0.47945 |
| nan nan | nan | IC_DIGITAL_COMPLEX | 1 | 130.0 | 479.5 | — | TI / Microchip – Digital IC Reliability | 0.47945 | 0.47945 |
| סה"כ מערכת |  |  |  |  |  |  |  |  | 30.91554 |

סה"כ λ_sys (יצרנים) = 30.9155 כשלים/10⁶ שע  (30915.5 FIT)  →  MTBF = 32,346 שעות  (3.7 שנים)

# 7  טבלת השוואה

הטבלה שלהלן מציגה השוואה בין שני הניתוחים לכל רכיב. יחס > 1 מציין שMIL-217F שמרני יותר מנתוני היצרן.

| תיאור / Part# | סוג | כמ' | λ_p MIL-217F (יחידה) | λ_p יצרן (יחידה) | יחס MIL/יצרן | λ_total MIL-217F | λ_total יצרן | הפרש % |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SMT CHIP JUMPER RESISTOR 0 Ohm,0.06 CRCW04020000ZSTD | RESISTOR | 41 | 0.00080 | 0.00529 | 0.15x | 0.03264 | 0.21693 | -85.0% |
| SMT CHIP RESISTOR 1 KOhm,1%,0.063W, CRCW04021K00FKTD | RESISTOR | 26 | 0.00080 | 0.00529 | 0.15x | 0.02070 | 0.13757 | -85.0% |
| SMT CHIP RESISTOR 75 Ohm,1%,0.25W,T CRCW120675R0FKTA | RESISTOR | 2 | 0.00080 | 0.01162 | 0.07x | 0.00159 | 0.02323 | -93.1% |
| SMT CHIP RESISTOR 100 KOhm,1%,0.063 CRCW0402100KFKTD | RESISTOR | 20 | 0.00080 | 0.00529 | 0.15x | 0.01592 | 0.10582 | -85.0% |
| SMT CHIP RESISTOR 49.9 Ohm,1%,0.063 CRCW040249R9FKTD | RESISTOR | 4 | 0.00080 | 0.00529 | 0.15x | 0.00318 | 0.02116 | -85.0% |
| SMT CHIP RESISTOR 200 KOhm,1%,0.063 CRCW0402200KFKTD | RESISTOR | 1 | 0.00088 | 0.00529 | 0.17x | 0.00088 | 0.00529 | -83.4% |
| SMT CHIP RESISTOR 24.9 KOhm,1%,0.06 CRCW040224K9FKTD | RESISTOR | 1 | 0.00080 | 0.00529 | 0.15x | 0.00080 | 0.00529 | -85.0% |
| SMT CHIP RESISTOR 48.7 KOhm,1%,0.06 CRCW040248K7FKTD | RESISTOR | 2 | 0.00080 | 0.00529 | 0.15x | 0.00159 | 0.01058 | -85.0% |
| CHIP RESIS 56.2KOHM 1% .063W T&R CRCW040256K2FKTD | IC | 2 | 0.25900 | 0.00529 | 48.95x | 0.51800 | 0.01058 | +4795.1% |
| SMT CHIP RESISTOR 402 Ohm,1%,0.063W CRCW0402402RFKTD | RESISTOR | 2 | 0.00080 | 0.00529 | 0.15x | 0.00159 | 0.01058 | -85.0% |
| SMT CHIP RESISTOR 40.2 KOhm,1%,0.06 CRCW040240K2FKTD | RESISTOR | 2 | 0.00080 | 0.00529 | 0.15x | 0.00159 | 0.01058 | -85.0% |
| CHIP RESIS 150 OHM 1% .063W T&R CRCW0402150RFKTD | IC | 3 | 0.25900 | 0.00529 | 48.95x | 0.77700 | 0.01587 | +4795.1% |
| SMT CHIP RESISTOR 2.21 KOhm,1%,0.06 CRCW04022K21FKTD | RESISTOR | 2 | 0.00080 | 0.00529 | 0.15x | 0.00159 | 0.01058 | -85.0% |
| SMT CHIP RESISTOR 4.7 KOhm,1%,0.063 CRCW04024K70FKTD | RESISTOR | 3 | 0.00080 | 0.00529 | 0.15x | 0.00239 | 0.01587 | -85.0% |
| SMT CHIP RESISTOR 137 KOhm,1%,0.063 CRCW0402137KFKTD | RESISTOR | 6 | 0.00088 | 0.00529 | 0.17x | 0.00526 | 0.03175 | -83.4% |
| SMT CHIP CAPACITOR 10000pF,10%,50V, C0402C103K5RALTU | CAPACITOR | 6 | 0.00221 | 0.00288 | 0.77x | 0.01329 | 0.01729 | -23.1% |
| CHIP CAP 0.1UF 10% 16V X7R T&R GRM033Z71C104KE14D | CAPACITOR | 199 | 0.00335 | 0.00288 | 1.16x | 0.66705 | 0.57332 | +16.3% |
| SMT CHIP RESISTOR 10 KOhm,1%,0.063W CRCW040210K0FKED | RESISTOR | 51 | 0.00080 | 0.00529 | 0.15x | 0.04060 | 0.26984 | -85.0% |
| SMT CHIP RESISTOR 2 KOhm,1%,0.063W, CRCW04022K00FKED | RESISTOR | 1 | 0.00080 | 0.00529 | 0.15x | 0.00080 | 0.00529 | -85.0% |
| SMT CHIP RESISTOR 499 Ohm,1%,0.063W CRCW0402499RFKED | RESISTOR | 7 | 0.00080 | 0.00529 | 0.15x | 0.00557 | 0.03704 | -85.0% |
| SMT CHIP RESISTOR 49.9 KOhm,1%,0.06 CRCW040249K9FKED | RESISTOR | 1 | 0.00080 | 0.00529 | 0.15x | 0.00080 | 0.00529 | -85.0% |
| SMT CHIP RESISTOR 42.2 KOhm,1%,0.06 CRCW040242K2FKED | RESISTOR | 2 | 0.00080 | 0.00529 | 0.15x | 0.00159 | 0.01058 | -85.0% |
| SMT CHIP RESISTOR 6.19 KOhm,1%,0.06 CRCW04026K19FKED | RESISTOR | 1 | 0.00080 | 0.00529 | 0.15x | 0.00080 | 0.00529 | -85.0% |
| Resistor SMT, 3.01Kohm, 1%, 0.063W, CRCW04023K01FKED | RESISTOR | 2 | 0.00080 | 0.00529 | 0.15x | 0.00159 | 0.01058 | -85.0% |
| SMT CHIP RESISTOR 23.2 Kohm,1%,0.06 CRCW040223K2FKTD | RESISTOR | 3 | 0.00080 | 0.00529 | 0.15x | 0.00239 | 0.01587 | -85.0% |
| SMT CHIP RESISTOR 8.45 KOhm,1%,0.06 CRCW04028K45FKTD | RESISTOR | 1 | 0.00080 | 0.00529 | 0.15x | 0.00080 | 0.00529 | -85.0% |
| SMT CHIP RESISTOR 10.7 Kohm,1%,0.06 CRCW040210K7FKTD | RESISTOR | 2 | 0.00080 | 0.00529 | 0.15x | 0.00159 | 0.01058 | -85.0% |
| SMT CHIP RESISTOR 130 KOhm,1%,0.063 CRCW0402130KFKTD | RESISTOR | 1 | 0.00088 | 0.00529 | 0.17x | 0.00088 | 0.00529 | -83.4% |
| SMT CHIP RESISTOR 150 KOhm,1%,0.063 CRCW0402150KFKTD | RESISTOR | 2 | 0.00088 | 0.00529 | 0.17x | 0.00175 | 0.01058 | -83.4% |
| SMT CHIP RESISTOR 69.8 KOhm,1%,0.06 CRCW040269K8FKTD | RESISTOR | 1 | 0.00080 | 0.00529 | 0.15x | 0.00080 | 0.00529 | -85.0% |
| CHIP RESIS 80.6KOHM 1^ .063W T&R CRCW040280K6FKTD | IC | 2 | 0.25900 | 0.00529 | 48.95x | 0.51800 | 0.01058 | +4795.1% |
| SMT CHIP RESISTOR 20.5 KOhm,1%,0.06 CRCW040220K5FKTD | RESISTOR | 2 | 0.00080 | 0.00529 | 0.15x | 0.00159 | 0.01058 | -85.0% |
| CHIP RESIS 44.2KOHM 1% .063W T&R CRCW040244K2FKTD | IC | 2 | 0.25900 | 0.00529 | 48.95x | 0.51800 | 0.01058 | +4795.1% |
| SMT CHIP RESISTOR 4.87 KOhm,1%,0.06 CRCW04024K87FKTD | RESISTOR | 1 | 0.00080 | 0.00529 | 0.15x | 0.00080 | 0.00529 | -85.0% |
| CHIP RESIS 33.2 OHM 1% .063W T/R CRCW040233R2FKED | IC | 52 | 0.25900 | 0.00529 | 48.95x | 13.46805 | 0.27513 | +4795.1% |
| CHIP RESIS 18.0 KOHM 0.1% 0.063W T& TNPW040218K0BETD | IC | 4 | 0.25900 | 0.00397 | 65.27x | 1.03600 | 0.01587 | +6427.2% |
| CHIP RESIS 59.7 KOHM 0.1% 0.063W T& TNPW040259K7BETD | IC | 2 | 0.25900 | 0.00397 | 65.27x | 0.51800 | 0.00794 | +6427.2% |
| SMT CHIP RESISTOR 4.75 KOhm,1%,0.06 CRCW04024K75FKED | RESISTOR | 50 | 0.00080 | 0.00529 | 0.15x | 0.03980 | 0.26455 | -85.0% |
| SMT CHIP CAPACITOR 1000pF,10%,100V, C0402C102K1GALTU | CAPACITOR | 1 | 0.00146 | 0.00288 | 0.51x | 0.00146 | 0.00288 | -49.2% |
| Resistor 1.96 Kohm, 1%, 0.063W, T&R CRCW04021K96FKED | RESISTOR | 1 | 0.00080 | 0.00529 | 0.15x | 0.00080 | 0.00529 | -85.0% |
| CHIP RESISTOR 71.5KOHM 1% .063W T&R CRCW040271K5FKED | RESISTOR | 1 | 0.00080 | 0.00529 | 0.15x | 0.00080 | 0.00529 | -85.0% |
| CHIP RESIS 11KOHM 1^ .063W T&R CRCW040211K0FKED | IC | 2 | 0.25900 | 0.00529 | 48.95x | 0.51800 | 0.01058 | +4795.1% |
| SMT CHIP CAPACITOR HIGH-VOLTAGE 100 C1206C102KGRACTU | CAPACITOR | 1 | 0.00146 | 0.00576 | 0.25x | 0.00146 | 0.00576 | -74.6% |
| CHIP CAP 220PF 2% 50V COG T&R C0402C221G5GACTU | CAPACITOR | 24 | 0.00111 | 0.00288 | 0.39x | 0.02674 | 0.06914 | -61.3% |
| SMT CHIP CAPACITOR 100pF,2%,100V,C0 C0402C101G1GACTU | CAPACITOR | 4 | 0.00097 | 0.00288 | 0.34x | 0.00387 | 0.01152 | -66.4% |
| CHIP CAP 4700PF 10% 100V X7R T&R C0603C472K1RACTU | CAPACITOR | 4 | 0.00193 | 0.00288 | 0.67x | 0.00773 | 0.01152 | -32.9% |
| SMT CHIP CAPACITOR 10uF,10%,25V,X7R C1206C106K3RACTU | CAPACITOR | 34 | 0.00768 | 0.00288 | 2.67x | 0.26112 | 0.09795 | +166.6% |
| 3.3V RS485 TRANSCEIVER WITH INTEGRA LTC2855HDE#PBF | IC | 6 | 0.25900 | 0.25817 | 1.00x | 1.55401 | 1.54900 | +0.3% |
| SMT ULTRA LOW CAPACITANCE TRANSIENT RCLAMP0531T.TCT | IC | 4 | 0.25900 | 0.02530 | 10.24x | 1.03600 | 0.10119 | +923.8% |
| SMT TRANSFORMER 10/100BASE-TX SINGL 100B-1003XNL | INDUCTOR | 1 | 0.01230 | 0.09622 | 0.13x | 0.01230 | 0.09622 | -87.2% |
| CHIP CAP 0.1UF 10% 25VOLT 40X20 X7R 04023C104KAT2A | CAPACITOR | 46 | 0.00335 | 0.00288 | 1.16x | 0.15419 | 0.13253 | +16.3% |
| LED BLUE 0402 SMD VLMB1500-GS08 | DIODE | 2 | 0.05273 | 0.07112 | 0.74x | 0.10546 | 0.14225 | -25.9% |
| CHIP CAP 0.1UF 10% 100V X7R T&R 06031C104K4T2A | CAPACITOR | 8 | 0.00335 | 0.00288 | 1.16x | 0.02682 | 0.02305 | +16.3% |
| DIODE TVS , 440W, 5.5V Bidirectiona D5V0S1B2LP | DIODE | 24 | 0.05273 | 0.02530 | 2.08x | 1.26554 | 0.60713 | +108.4% |
| CHIP COMMON MODE CHOKE COIL 390OHM DLW21SH391XQ2L | IC | 12 | 0.25900 | 0.01162 | 22.30x | 3.10801 | 0.13938 | +2129.9% |
| IC DC/DC STEP DOWN REGULATOR 8A 20V LTM4657IY | IC | 2 | 0.12845 | 1.03266 | 0.12x | 0.25689 | 2.06533 | -87.6% |
| OSC 25.0 MHz -40 to +105 625L3G25M00000 | IC | 2 | 0.25900 | 0.18973 | 1.36x | 0.51800 | 0.37945 | +36.5% |
| CHIP RESISTOR 34.8 Kohm 1% 0.063W T CRCW040234K8FKED | RESISTOR | 1 | 0.00080 | 0.00529 | 0.15x | 0.00080 | 0.00529 | -85.0% |
| CHIP RESIS 1KOHM 1% .75W 1206 HP CRCW12061K00FKEAHP | IC | 8 | 0.25900 | 0.00529 | 48.95x | 2.07201 | 0.04233 | +4795.1% |
| CHIP EMI FILTER FERRITE BEAD 120 Oh BLM15PX121SN1D | IC | 29 | 0.25900 | 0.00581 | 44.59x | 7.51103 | 0.16843 | +4359.4% |
| SMT CHIP EMI FILTER FERRITE BEAD 22 BLM18KG221SN1D | IC | 6 | 0.25900 | 0.00581 | 44.59x | 1.55401 | 0.03485 | +4359.4% |
| SMT CHIP CAPACITOR1uF,10%,50V,X7R,T 06035C105KAT2A | CAPACITOR | 23 | 0.00507 | 0.00288 | 1.76x | 0.11670 | 0.06626 | +76.1% |
| SINGLE 2 INPUT POSITIVE AND GATE SN74LVC1G08QDRYRQ1 | IC | 2 | 0.25900 | 0.07376 | 3.51x | 0.51800 | 0.14752 | +251.1% |
| SMT HCMOS CRYSTAL TCXO 40 MHz,3.3V, 536EL400X2GT5 | CRYSTAL | 1 | 0.15115 | 0.35837 | 0.42x | 0.15115 | 0.35837 | -57.8% |
| SMT CHIP CAPACITOR 10uF,10%,10V,X7R GRM188Z71A106KA73D | CAPACITOR | 148 | 0.00768 | 0.00288 | 2.67x | 1.13664 | 0.42639 | +166.6% |
| SMT 10/100 Mbps ETHERNET PHYSICAL L DP83822HRHBR | IC | 1 | 0.25900 | 0.47945 | 0.54x | 0.25900 | 0.47945 | -46.0% |
| SMT CHIP CAPACITOR 22uF,10%,10V,X7R C1206C226K8RACTU | CAPACITOR | 44 | 0.00885 | 0.00288 | 3.07x | 0.38944 | 0.12676 | +207.2% |
| SERIAL NOR FLASH 1GB 1.8V MT25QU01GBBB8E12-0AUT | IC | 1 | 0.25900 | 0.27661 | 0.94x | 0.25900 | 0.27661 | -6.4% |
| CHIP CAP CER 1UF 10% 10V X7R 0402 T CC0402KRX7R6BB105 | CAPACITOR | 211 | 0.00507 | 0.00288 | 1.76x | 1.07061 | 0.60789 | +76.1% |
| CHIP CAP 47UF 20^ 10V X7R T&R GRM32ER71A476ME15L | CAPACITOR | 1 | 0.01015 | 0.00288 | 3.52x | 0.01015 | 0.00288 | +252.2% |
| CHIP RESIS 499 OHM 0.1^ 0.063W T&R TNPW0402499RBEED | IC | 1 | 0.25900 | 0.00529 | 48.95x | 0.25900 | 0.00529 | +4795.1% |
| TDA4VEN PROCESSOR 18X18MM 594FCBGA TDA4VEN8K5AAMWRQ1 | IC | 1 | 4.34145 | 1.65964 | 2.62x | 4.34145 | 1.65964 | +161.6% |
| AGILEX5 E FPGA NO HPS 656K 23X23MM A5EC065BB23AI6X | IC | 1 | 5.21252 | 3.31927 | 1.57x | 5.21252 | 3.31927 | +57.0% |
| Connector PCI Express PCIe M2 Conne MDT275M01001 | CONNECTOR | 1 | 1.77977 | 0.03703 | 48.06x | 1.77977 | 0.03703 | +4705.6% |
| LPDDR4 SDRAM 2GB(16Gb) AUTOMOTIVE U MT53E512M32D1ZW-046AUT:B | IC | 1 | 0.25900 | 0.33193 | 0.78x | 0.25900 | 0.33193 | -22.0% |
| DC/DC 20VIN 20A STEP-DOWN REGULATOR LTM4640IY#PBF | IC | 2 | 0.12845 | 1.03266 | 0.12x | 0.25689 | 2.06533 | -87.6% |
| 16VIN 4A SILENT SWITCHER REGULATOR LTM4732IY#PBF | IC | 7 | 0.12845 | 1.03266 | 0.12x | 0.89911 | 7.22864 | -87.6% |
| 20V 1A ULTRA LOW NOISE-PSRR LINEAR LT3041ADE#PBF | IC | 3 | 0.12845 | 0.20284 | 0.63x | 0.38533 | 0.60854 | -36.7% |
| NOR FLASH OSPI 2Gbit 1.8V AUTOMOTIV MT35XU02GCBA1G12-A0AUT | IC | 1 | 0.25900 | 0.27661 | 0.94x | 0.25900 | 0.27661 | -6.4% |
| ETHERNET SWITCH 5 PORTS WITH 4X 100 LAN9370-V/KCXVAO | IC | 1 | 0.25900 | 0.47945 | 0.54x | 0.25900 | 0.47945 | -46.0% |
| HCMOS CLOCK OSCILLATOR 1.8V 25MHZ 5 625M3G025M00000 | CRYSTAL | 4 | 0.15115 | 0.18973 | 0.80x | 0.60459 | 0.75891 | -20.3% |
| LPDDR4 SDRAM 4GB(32Gb) AUTOMOTIVE T MT53E1G32D2FW-046AAT:C | IC | 1 | 0.25900 | 0.33193 | 0.78x | 0.25900 | 0.33193 | -22.0% |
| CAP EMI FILTER 3TERM 1UF 4V 0402 20 NFM15HC105D0G3D | CAPACITOR | 42 | 0.00507 | 0.00581 | 0.87x | 0.21311 | 0.24394 | -12.6% |
| SINGLE GMSL2 TO MIPI CSI-2 DESERIAL MAX96714GTJ/VY+T | IC | 2 | 0.25900 | 0.73762 | 0.35x | 0.51800 | 1.47523 | -64.9% |
| FILTER EMI MLC 10UF 20% 4V 2A 0603 NFM18HC106D0G3L | IC | 10 | 0.25900 | 0.00346 | 74.92x | 2.59001 | 0.03457 | +7392.1% |
| SMT MICRO SOCKET 1.27MMX2.54MM 20PI ASP-248396-02 | IC | 1 | 4.34145 | 0.03703 | 117.22x | 4.34145 | 0.03703 | +11622.5% |
| Connector to EOS SEAF-20-01-L-04-2-RA-GP-T | CONNECTOR | 1 | 1.77977 | 0.03703 | 48.06x | 1.77977 | 0.03703 | +4705.6% |
| Connector to DLT G125-MS12605M2P | CONNECTOR | 1 | 1.77977 | 0.03703 | 48.06x | 1.77977 | 0.03703 | +4705.6% |
| Connector to UMBILI G125-MS13405M2P | CONNECTOR | 1 | 1.77977 | 0.03703 | 48.06x | 1.77977 | 0.03703 | +4705.6% |
| Connector Board to Board 40P 2X20 0 QTE-020-05-L-D-A-K | CONNECTOR | 1 | 1.77977 | 0.03703 | 48.06x | 1.77977 | 0.03703 | +4705.6% |
| CHIP RESIS 6.49 KOHM 0.1% 0.063W 04 TNPW04026K49BEED | IC | 2 | 0.25900 | 0.00397 | 65.27x | 0.51800 | 0.00794 | +6427.2% |
| 1210 COMMON MODE CHOKE COIL 200UH 7 DLW32MH201XK2L | IC | 4 | 0.25900 | 0.01162 | 22.30x | 1.03600 | 0.04646 | +2129.9% |
| FRAM 2MBIT SPI 50MHZ 8X256K 1.8V DF CY15V102QN-50LHXI | IC | 1 | 0.25900 | 0.14752 | 1.76x | 0.25900 | 0.14752 | +75.6% |
| ADJUSTABLE SEQUENCING/SUPERVISORY C MAX16895AALT+T | IC | 1 | 0.25900 | 0.16596 | 1.56x | 0.25900 | 0.16596 | +56.1% |
| CHIP RESIS 240 OHM 1% .063W 0402 T& CRCW0402240RFKED | IC | 7 | 0.25900 | 0.00529 | 48.95x | 1.81301 | 0.03704 | +4795.1% |
| SMT CHIP RESISTOR 12 KOhm,1%,0.063 CRCW040212K0FKTD | RESISTOR | 1 | 0.00080 | 0.00529 | 0.15x | 0.00080 | 0.00529 | -85.0% |
| SMT CHIP CAPACITOR 2.2uF,10%,10V,X7 C0603C225K8RACTU | CAPACITOR | 4 | 0.00585 | 0.00288 | 2.03x | 0.02339 | 0.01152 | +103.0% |
| CAP CHIP 0.01UF 10% 25V X7R 0201 T& GRM033R71E103KE14D | CAPACITOR | 10 | 0.00221 | 0.00288 | 0.77x | 0.02215 | 0.02881 | -23.1% |
| RES SMD 100K OHM 1% 0.33W 0603 CRCW0603100KFKEAHP | RESISTOR | 4 | 0.00080 | 0.01162 | 0.07x | 0.00318 | 0.04646 | -93.1% |
| nan nan | IC | 1 | 0.25900 | 0.47945 | 0.54x | 0.25900 | 0.47945 | -46.0% |
| nan nan | IC | 1 | 0.25900 | 0.47945 | 0.54x | 0.25900 | 0.47945 | -46.0% |
| סה"כ מערכת |  | — | — | — | 2.46x | 76.14182 | 30.91554 | +146.3% |

# 8  MTBF מסכם ומסקנות

## 8.1  תוצאות מסכמות – השוואה

| פרמטר | MIL-STD-217F | נתוני יצרנים |
| --- | --- | --- |
| λ_sys (כשלים/10⁶ שע) | 76.1418 | 30.9155 |
| FIT כולל (כשלים/10⁹ שע) | 76141.8 | 30915.5 |
| MTBF (שעות) | 13,133 | 32,346 |
| MTBF (שנים) | 1.5 | 3.7 |

## 8.2  פרשנות

יחס MIL-217F / יצרנים: 2.46x – ניתוח MIL-217F שמרני ביחס לנתוני היצרנים.

ה-MTBF הנמוך מבין שני הניתוחים הוא 13,133 שעות (1.5 שנים) – מצביע על צורך בשיפור אמינות: שדרוג איכות רכיבים, הפחתת טמפרטורה, ו/או Derating.

## 8.3  המלצות

• זיהוי הרכיבים בעלי λ_p הגבוה ביותר (Top 10) וביצוע Derating לפחות ל-70%

• ביצוע Thermal Management לרכיבים בעלי π_T גבוה

• שדרוג רמת האיכות לרכיבים קריטיים מרמה C לרמה B-2 ומעלה

• ביצוע Burn-In להפחתת כשלות ינקות (Infant Mortality)

• בחינת רכיבים שיחס MIL-217F/יצרן > 5x – ייתכן שניתן לשפר הנחות

• עדכון ניתוח זה בכל שינוי ב-BOM, בתנאי הסביבה, או ברמת האיכות

──────────────────────────────────────────────────────────────────────

מסמך זה הופק אוטומטית. יש לאמת את הנחות הקלט לפני שימוש בתוצאות.